Smart Greenhouse Monitor with Custom Queries & Intervals Using a Microbit v2

Technical Architecture & System Design Report

Made for EGC-121 Computer Architecture  @IIIT-Bangalore in April 2025

| PLATFORM nRF52833 | ISA Thumb-2 | CLOCK 64 MHz |
| --- | --- | --- |

| FSM STATES 6 | QUERY TIME O(log N) | LANG ARM ASM |
| --- | --- | --- |

Made by: Pratham Vyas (BC2025078)  &
                Yashamit Dawane (BC2025119)

Deploying this bare-metal data logger in a commercial greenhouse transforms it from a technical exercise into a highly practical agricultural tool. (A real greenhouse temperature logger would update less frequently than 5 seconds interval used here for demo purposes.) The underlying "Triple Segment Tree" architecture is particularly valuable in this context; instead of forcing a grower to export and analyze hundreds of raw data points, the monitor can instantly compute the exact daily Maximum (to check for plant heat stress), Minimum (to ensure frost protection limits weren't breached), and Average temperatures (to calculate Growing Degree Days for harvest timing). Coupled with the robust hardware failsafes engineered into the system, this architecture provides low cost, bulletproof, remote climate tracking without the fragility, cost or power drain of a full operating system.

Computer Architecture Project  |  ARM Cortex-M4  |  Bare-Metal Assembly

# 1. Executive Summary

This report documents the complete engineering lifecycle of a bare-metal embedded system deployed on the ARM Cortex-M4 (nRF52833) microcontroller. The system continuously acquires, buffers, and statistically analyzes environmental temperature data with zero operating system overhead, achieving deterministic real-time performance across all subsystems.

The architecture is defined by three principal innovations: a 6-state Finite State Machine (FSM) governing all control flow; asynchronous Event Latch architecture; a custom Persistence of Vision (POV) multiplexing engine for the 5×5 LED matrix display; and a "Triple Segment Tree" — three independent segment trees (Min, Max, Sum) operating in concert to deliver O(log N) range queries across a 128-element ring buffer. The entire system is implemented in ARM Assembly (Thumb-2 ISA) within a single monolithic translation unit.

## System Specifications at a Glance

| Parameter | Value / Specification |
| --- | --- |
| Platform | ARM Cortex-M4 (nRF52833) |
| Clock Speed | 64 MHz |
| ISA | Thumb-2 (ARMv7E-M) |
| Language | 100% ARM Assembly (.S) |
| OS / RTOS | None (Bare-Metal) |
| Data Structure | Triple Segment Tree (Min/Max/Sum) |
| Query Complexity | O(log N) ¦ N = 128 leaves |
| FSM States | 6 (Idle → Record → Query → L-Bound → R-Bound → Result) |
| Display | 5×5 LED Matrix (nRF52833 GPIO) |
| Display Algorithm | Sliding Window Marquee (11-bit virtual frame buffer) |
| Sensor Interface | I2C (bit-banged driver) |
| Debounce Strategy | Software busy-wait (guaranteed single-event resolution) |
| Input Architecture | Asynchronous RAM Latch (Smart Polling during blocking delays) |

# 2. System Architecture & Collaborative Design

## 2.1  Monolithic Assembly Strategy

To minimize linker complexity and enable rapid prototyping under strict project deadlines, the system was implemented within a single monolithic main.S file. This approach eliminates inter-object-file linkage overhead but introduces significant coordination risk in a multi-developer environment: bare-metal assembly provides no namespace isolation, no scoping primitives, and no type system. Any label defined at file scope is globally visible, making symbol collisions both trivial to create and catastrophic in their effects.

## 2.2  Collision Mitigation Protocol

Three architectural controls were established to enforce isolation within the shared file:

- Immutable Territory Division — The file was partitioned into three non-overlapping zones: .data (static memory allocation), fsm_loop (top-level control flow dispatch), and subroutines (isolated functional units). No code was permitted to span zone boundaries.

- Single-Feature Subroutine Rule — Each subroutine was required to implement exactly one logical feature. Cross-feature register usage was forbidden without explicit documentation.

- Namespace Prefixing Convention — All labels were required to carry a feature-prefix (e.g., debounce_loop, i2c_read_byte, tree_insert_leaf) to eliminate accidental collisions. The first integration build surfaced a multiply defined symbol error on delay_loop (independently authored by both developers); the namespace policy resolved this within minutes.

## 2.3  Layer-Based Labor Division

Development responsibilities were partitioned by architectural layer to minimize merge conflicts:

| Developer | Layer | Ownership |
| --- | --- | --- |
| Pratham | Backend / Data | I2C sensor driver, Triple Segment Tree math engine, FSM State 1 (Record), Event Latching |
| Yashamit | Frontend / UI | FSM states 2–5, 5×5 LED multiplexer, POV sliding-window marquee algorithm, Event Latching |

# 3. Finite State Machine (FSM) Control Flow

The system's runtime behavior is governed by a deterministic 6-state FSM. Transitions are driven exclusively by debounced button events; no state transition can occur due to sensor activity, display activity, or timer overflow. This strict separation of concerns ensures that the control plane is fully isolated from the data plane.

## 3.1  State Transition Diagram

The FSM executes a linear pipeline from data acquisition to result display, with bounded navigation loops in the query-selection states:

| ID | STATE | DESCRIPTION |
| --- | --- | --- |
| STATE 0 | IDLE | Awaiting user input System in low-power poll |
| STATE 1 | RECORD | I2C sensor read Data written to segment tree |
| STATE 2 | QUERY SELECT | User selects query type (Min / Max / Avg) |
| STATE 3 | L-BOUND | Left bound selection Cursor navigation active |
| STATE 4 | R-BOUND | Right bound selection Range validation |
| STATE 5 | RESULT | O(log N) tree traversal POV marquee display |

Transition direction: IDLE → RECORD → QUERY SELECT → L-BOUND → R-BOUND → RESULT → IDLE. Each left-arrow transition (result display → idle) completes one full acquisition-and-query cycle.

## 3.2  Software Debouncing

Mechanical push-buttons exhibit contact bounce — a burst of transient open/close events occurring within approximately 5–20 ms of a physical actuation. Without mitigation, a single button press can generate dozens of spurious state transitions.

The implemented solution is a software busy-wait debouncer inserted after every state transition:

The loop counter is calibrated to the processor clock rate, producing a dead-zone of approximately 20 ms — sufficient to mask all bounce transients while remaining imperceptible to the user.

# 4. Data Structure: The Triple Segment Tree

## 4.1  Design Rationale

Continuous sensor acquisition demands a data structure capable of answering three distinct range queries — minimum, maximum, and average — over arbitrary subranges of the acquisition buffer, without re-scanning the raw data on every query. A naive linear scan would produce O(N) query time, which at N=128 samples would visibly stall the display update loop at 64 MHz. The Segment Tree reduces this to O(log N) = O(7) comparisons per query, ensuring that even a full-range query over all 128 samples completes in negligible processor time.

Three independent segment trees are maintained in parallel — one for minimum, one for maximum, and one for sum — sharing a single logical cursor and insertion path. This "Triple Segment Tree" pattern avoids the overhead of three separate traversals during insertion; all three trees are updated in one O(log N) upward sweep.

## 4.2  Memory Layout & Indexing

| ROOT [1] | Stores global Min / Max / Sum across all 128 leaves. Updated on every insertion via O(log N) bubble-up. |
| --- | --- |
| INTERNAL NODES [2..127] | Each internal node aggregates its two children. Parent lookup: node >> 1 (single LSR instruction). Child lookup: node << 1 (single LSL instruction). |
| LEAF NODES [128..255] | 128 physical data leaves. cursor_index advances via ring-buffer modulo (AND 0x7F). Offset computed: base_addr + (leaf_idx << 2). |

## 4.3  Key Assembly Optimizations

- LSR Parent traversal: a single LSR (Logical Shift Right) instruction computes parent(n) = n >> 1, exploiting the 1-based indexing property of binary heaps.

- LSL Child traversal: a single LSL (Logical Shift Left) instruction computes left_child(n) = n << 1.

- Offset Leaf address calculation: the byte offset of any leaf is computed as base_addr + (leaf_idx << 2), using the processor's barrel shifter — no multiplication required.

- UDIV Average computation: the hardware udiv instruction divides the sum tree query result by the node count of the selected range, computed from the query bounds.

- @ --- O(log N) TREE TRAVERSAL VIA BIT-SHIFTING --- - bubble_max: - lsr r5, r5, #1 @ Parent Index = Child / 2 - cmp r5, #0 - beq bubble_max_done - lsl r4, r5, #1 @ Left Child Index = Parent * 2 - lsl r7, r4, #2 @ Multiply by 4 for byte offset - ldr r8, [r3, r7] @ Load Left Child - add r4, r4, #1 @ Right Child Index = (Parent * 2) + 1 - lsl r7, r4, #2 - ldr r9, [r3, r7] @ Load Right Child

## 4.4  Phase 1 Bug: Hardcoded Leaf Insertion

The initial implementation hardcoded the insertion target to memory offset #16 (Leaf 0), producing a system where the root node appeared to update correctly while Leaves 1–127 contained static uninitialized data. The failure mode was subtle: the display showed plausible temperature readings that never changed regardless of environmental variation.

Diagnosis was performed using GDB attached via OpenOCD with a hardware breakpoint on the math subroutine entry point:

(gdb) x/10wx &tree_array

; Result: root node = dynamic, leaves[1..3] = 0xDEADBEEF (uninitialized)

The fix introduced a dynamic cursor_index register (r6) incremented on each insertion and wrapped via bitwise AND (AND r6, #0x7F) to implement O(1) ring-buffer behavior. The leaf byte offset is then computed as lsl r7, r6, #2.

# 5. Display Engineering: LED Matrix & POV

## 5.1  Hardware Multiplexing

The nRF52833's integrated 5×5 LED matrix cannot drive all 25 LEDs simultaneously without exceeding the GPIO current budget. The hardware requires time-division multiplexing: each of the 5 rows is powered sequentially for a brief interval, with only the LEDs in that row illuminated. At sufficient refresh rate, persistence of vision causes the display to appear fully lit.

The multiplexer was implemented entirely in ARM Assembly, with no hardware timer support — all timing is achieved via calibrated instruction-count busy-wait loops.

## 5.2  Hardware Race Condition: Ghosting

The initial multiplexer implementation exhibited "ghosting" — dim, incorrectly-positioned LED activations that persisted into subsequent row cycles. Root cause analysis identified a GPIO race condition: the Row (anode) signal was driven HIGH before the Column (cathode) data was settled on the GPIO pins. Because the Cortex-M4 executes at 64 MHz, the Row activated while the previous frame's column data was still propagating, causing momentary incorrect LED activations.

@ --- GHOSTING FIX: COLUMN-FIRST SEQUENCING --- show_digit_row: @ 1. SET COLUMNS FIRST (Prevents previous row's data from bleeding) ldr r9, [r6, r7, lsl #2] mvn r0, r9 and r0, r0, #0b11111 bl write_column_pins @ 2. NOW ACTIVATE THE ROW mov r8, #1 lsl r8, r8, r7 mov r0, r8 bl write_row_pins

## 5.3  Lateral Inversion & Font Table

Asymmetric digit glyphs (2, 3, 5, etc.) rendered as mirror images. Hardware documentation review revealed that the GPIO port maps Bit 0 to Column 0 (the physical left edge of the matrix), contrary to the assumed right-to-left bit ordering. The entire 200-byte font_table was reauthored to respect the physical pin-to-column mapping, producing correct left-aligned block-letter glyphs.

## 5.4  Sliding Window Marquee Algorithm

Rendering two-digit results (e.g., 24°C) by sequentially flashing the tens and ones digits produced an unreadable overlap artifact. The 64 MHz processor cycles through both digit frames faster than the human flicker-fusion threshold (~60 Hz), causing both glyphs to appear superimposed.

The Sliding Window Marquee algorithm was engineered as a replacement:

- The 5-column tens digit bitmap and the 5-column ones digit bitmap are concatenated, separated by a 1-column blank, into an 11-bit virtual frame buffer held in a CPU register.

- A dynamic scroll_offset variable (0..10) is incremented on each display refresh cycle.

- The current 5-column display window is extracted by applying a Logical Shift Right (LSR) of scroll_offset bits to the frame buffer, then masking the lower 5 bits for GPIO output.

- The result is a smooth ticker-tape scroll of the two-digit result across the 5×5 matrix — readable, artifact-free, and implemented with three assembly instructions per refresh.

- @ --- 11-BIT VIRTUAL FRAME BUFFER & SLIDING WINDOW --- - @ B. Build the invisible wide image in r9 - lsl r9, r9, #5 @ Push Tens digit 5 bits left - lsl r10, r10, #11 @ Push Ones digit 11 bits left (creates 1-bit gap) - orr r9, r9, r10 @ Bitwise OR to combine into single 16-bit register - @ C. Apply the Sliding Window - lsr r9, r9, r6 @ Shift the wide image right by dynamic 'scroll_offset' (r6) - and r9, r9, #0b11111 @ Mask out everything except the 5 active columns

# 6. Defect Registry & Resolution Log

The following table catalogues all significant defects identified during integration and validation, including the diagnostic methodology and final resolution applied:

| Defect | Root Cause Fix | Diagnostic Method | Severity |
| --- | --- | --- | --- |
| Floating GPIO Pins | PIN_CNF 0x0C — internal pull-up resistors on Pins 14 & 23 | Ambient EMI observation | Critical — spurious state changes |
| GPIOTE Interrupt Bounce | Replaced interrupts with polling + 250ms software debounce | Logic analysis / state trace | Critical — UI skipping states |
| Empty Query / Div-by-Zero | error_no_data guard at Recording state exit | HardFault trap observation | Critical — silent system halt |
| Inverted Range Bounds | R-Bound scroll minimum clamped to query_l value | Boundary condition testing | Major — OOB memory read |
| LED Ghosting (Race Condition) | Inverted GPIO sequence: Column-first then Row | Hardware logic analyzer | Severe — display unreadable |
| Lateral Digit Inversion | Rewrote 200-byte font_table per hardware pin mapping | Hardware schematic review | Major — numbers mirrored |
| Multiply-Defined Symbols | Namespace refactor: delay_loop → debounce_loop | Assembler error log | Build-breaking |
| Thumb-2 IT Block Violation | Replaced naked moveq with explicit ITE blocks | GAS assembler error | Build-breaking |
| POV Flicker Fusion | 11-bit marquee shift register with scroll offset | Visual inspection | Major — UX unusable |
| Unresponsive UI during POV Marquee | Refactored blocking delay_loop into an asynchronous RAM Event Latch | State transition timing analysis | Major — UX unusable |

# 7. Thumb-2 ISA Compliance & Instruction Set Strictness

## 7.1  Conditional Execution: IT Block Requirement

The ARM Cortex-M4 implements the Thumb-2 instruction set, which deprecates the ARMv4T naked conditional suffix model (e.g., moveq, addne) for most instructions. Conditional data-processing instructions must be wrapped in an explicit IT (If-Then) or ITE (If-Then-Else) block, which encodes the condition and predication mask in a dedicated 16-bit IT instruction preceding the conditional body.

The temperature clamping logic (constraining display values to [0, 99]) was refactored from naked conditional moves to compliant ITE blocks:

@ --- THUMB-2 COMPLIANT IT BLOCK --- @ Clamp value to 99 maximum cmp r4, #99 it gt @ IF Greater Than... movgt r4, #99 @ THEN move 99 into r4

## 7.2  Immediate Value Constraints

Thumb-2 imposes tight constraints on immediate operands in bitwise instructions. The bitmask 0x3FF (10-bit GPIO pin field) is not encodable as a valid Thumb-2 immediate in a TST or AND instruction; valid 8-bit shifted immediates must be used instead. The mask was reduced to 0xFF, isolating only the lower 8 GPIO pins relevant to the LED row/column drive signals, with no loss of functional correctness for the matrix dimensions in use.

## 7.3  Branch Instruction Compliance

Button polling was refactored from flag-conditional data instructions to explicit comparison-and-branch pairs using BEQ/BNE, which are fully compliant across all Thumb-2 implementations and require no IT block wrapping:

@ --- THUMB-2 COMPLIANT BRANCHING --- @ Original (Caused assembler errors due to naked conditional returns): @ tst r1, #(1 << 14) @ moveq r0, #1 @ bxeq lr @ Refactored (Explicit BEQ/BNE comparison-and-branch pairs): poll_buttons: ldr r0, =0x50000000 ldr r1, [r0, #0x510] @ Check Button A tst r1, #(1 << 14) beq pb_a_pressed @ Check Button B tst r1, #(1 << 23) beq pb_b_pressed @ Neither pressed mov r0, #0 bx lr pb_a_pressed: mov r0, #1 bx lr pb_b_pressed: mov r0, #2 bx lr

# 8. Hardware Interfacing:

In a high-level language, a button press is an abstracted boolean event. At the bare-metal hardware level, a button is a mechanical spring contact that vibrates at a microscopic scale upon actuation, generating a burst of electrical transients that an unprotected GPIO input registers as dozens of distinct digital events. Taming this behavior required two independent interventions targeting different failure modes.

## 8.1  The Floating Pin Disaster

| Failure | The FSM would spontaneously jump states with no physical button interaction. Ambient electromagnetic interference — proximity of a hand, a person walking past the desk — was sufficient to trigger state transitions. |
| --- | --- |
| Root Cause | The GPIO pins were left in a high-impedance floating state. Without a defined default voltage level, the pin acts as a small antenna, coupling ambient RF noise and electrostatic charge into the digital input threshold comparator and producing spurious logic-HIGH or logic-LOW events. |
| Fix | The init_hardware subroutine was rewritten to explicitly write 0x0C to the PIN_CNF (Pin Configuration) registers for Pins 14 and 23. This asserts the internal pull-up resistors, holding the pin at a stable HIGH (VDD) by default. Button actuation physically grounds the pin to VSS, producing a clean Active-LOW edge transition. |

@ --- HARDWARE PULL-UP CONFIGURATION --- init_hardware: @ Button A (Pin 14) ldr r0, =0x50000738 @ PIN_CNF[14] register mov r1, #0x0C @ 0x0C = Input Buffer Connect + Pull-Up Resistor str r1, [r0] @ Button B (Pin 23) ldr r0, =0x5000075C @ PIN_CNF[23] register mov r1, #0x0C str r1, [r0] bx lr

## 8.2  The Interrupt Trap & Switch Bounce

| Failure | The initial implementation used GPIOTE hardware interrupts for UI navigation. A single physical button press would frequently cause the FSM to skip two or three states simultaneously, bypassing menus entirely. |
| --- | --- |
| Root Cause | Mechanical switch bounce. Metal contact surfaces are not perfectly smooth; upon actuation, the contacts make and break electrical connection 10–20 times within a 1–5 ms window before settling. At 64 MHz, the processor can execute approximately 64,000 instructions per millisecond — easily resolving each individual bounce spike as a distinct interrupt edge, firing the ISR up to 15 times per physical press. |
| Fix | Hardware interrupts were abandoned for all UI navigation paths. A Polling Architecture with Software Debouncing was substituted. On detection of a button press (tst r1, #(1 << 14)), the FSM registers the action and immediately enters debounce_delay — a calibrated busy-wait loop producing approximately 250 ms of pin-state blindness, sufficient to outlast all mechanical bounce transients. |

## 8.3  The CPU Blind Spot & Event Latching

| Failure | During the SHOW_RESULT phase, the UI felt highly unresponsive. The user had to awkwardly hold Button A down for a full second to transition back to the IDLE state. Quick taps were completely ignored by the system. |
| --- | --- |
| Root Cause | Synchronous blocking delays. Because the architecture lacks a preemptive RTOS, the 64 MHz processor was trapped inside a 1-second delay_loop to hold the LED frame visible. During this time, the CPU was entirely blind to the physical GPIO pins. |
| Fix | Engineered an asynchronous Event Latch (Sticky Flag) architecture. The standard delay loop was refactored into a "Smart Poller." The CPU now interrupts its own delay countdown every 256 cycles to poll the GPIO pins. If a button press is detected, it writes an event code to a dedicated button_event RAM register and immediately aborts the delay. The master FSM polling logic was upgraded to consume this RAM latch before checking the physical pins, resulting in instantaneous, uniform response times for 10-millisecond taps across every state. |

@ --- SMART POLLING & EVENT LATCHING --- delay_loop: push {r1-r4, lr} mov r4, r0 @ r4 = delay count ldr r1, =0x50000000 @ GPIO Base dl_spin: subs r4, r4, #1 beq dl_done @ Smart Poll: Check physical pins every 256 cycles tst r4, #0xFF bne dl_spin @ Read GPIO IN register ldr r2, [r1, #0x510] @ Check Button A tst r2, #(1 << 14) beq dl_latch_a @ If pressed, branch to Latch! b dl_spin @ Else, continue delay dl_latch_a: ldr r3, =button_event @ Memory address of RAM latch mov r2, #1 str r2, [r3] @ Save "Button A Pressed" to RAM b dl_done @ ABORT delay loop early

# 9. Failsafes & Error Handling: Curing the "Silent Death"

An embedded system without an operating system, serial console, or exception-handling runtime has no conventional mechanism to surface runtime errors. An illegal operation — a division by zero, an out-of-bounds memory access, an unexpected FSM state — either triggers a processor HardFault exception (which halts the CPU permanently) or silently corrupts the output data. In either case, from the user's perspective, the device simply stops working with no diagnostic information.

Two distinct failure modes were identified during integration testing, each requiring a targeted defensive engineering response.

## 9.1  The "Empty Query" Bug — Division by Zero

| Failure | If the user powered on the device and immediately pressed Button A to bypass the recording phase, then requested an Average Query, the system would freeze permanently (silent HardFault). |
| --- | --- |
| Root Cause | With zero readings recorded, total_count = 0. The Segment Tree average computation issued udiv r0, r0, count with a zero divisor. The ARM Cortex-M4 udiv instruction with a zero divisor produces an UNPREDICTABLE result per the architecture reference manual; in practice, the processor entered a fault handler with no OS to recover execution. |
| Fix | A guard condition was inserted at the exit of the Recording state: if total_readings == 0, the FSM refuses to advance to the Query Menu. Additionally, a dedicated error_no_data trap was added to the average computation path as a secondary safety net. |

## 9.2  The "Time-Travel" Bug — Inverted Range Bounds

| Failure | During FSM States 3 and 4 (L-Bound and R-Bound selection), the user could configure Left Bound = 5 and Right Bound = 2, creating a temporally inverted query range. |
| --- | --- |
| Root Cause | The array traversal logic assumed Left Bound <= Right Bound invariantly. An inverted range caused the Segment Tree traversal pointers to cross, producing reads from memory addresses outside the allocated tree_array buffer — accessing uninitialized or foreign stack data. |
| Fix | The FSM logic for State 4 (R-Bound selection) was modified to clamp the Right Bound scroll minimum to the current value of query_l rather than to 0. This enforces the Left Bound <= Right Bound invariant at the UI level, making an inverted range geometrically impossible to construct. |

## 9.3  The Visual Watchdog: flash_error Subroutine

With no serial output or display console, error signaling must be physical. The flash_error subroutine serves as a hardware-level "Check Engine" light — a last-resort visual alarm that fires before any controlled system reset:

| # | Mechanism | Description |
| --- | --- | --- |
| 1 | Pre-emptive Menu Clamping | R-Bound selector wraps to query_l minimum instead of 0, enforcing Left <= Right at the input layer before any tree traversal occurs. |
| 2 | error_no_data Guard | Zero-reading check at Recording exit. FSM transition to Query Menu is unconditionally blocked if total_readings == 0. |
| 3 | flash_error Subroutine | On any trapped illegal state, all 25 LEDs are driven to maximum brightness and flashed four times in rapid succession before the system performs a controlled reset to FSM State 0 (IDLE). |

# 10. Technical Skills Demonstrated

| Skill Domain | Evidence |
| --- | --- |
| ARM Assembly (Thumb-2) | Complete system in .S — IT/ITE blocks, barrel shifter, LSL/LSR optimization, valid immediates |
| Embedded Systems Design | Bare-metal nRF52833 with no RTOS, direct register manipulation, GPIO timing |
| Data Structures & Algorithms | O(log N) Triple Segment Tree with ring-buffer cursor and dynamic address computation |
| Hardware Debugging | GDB + OpenOCD JTAG session, memory dump analysis (x/10wx), hardware breakpoints |
| Digital Logic / GPIO | Multiplexer design, race condition analysis, column-first GPIO sequencing fix |
| Algorithm Design | Sliding Window Marquee — 11-bit virtual frame buffer with shift-mask extraction |
| Collaborative Engineering | Namespace protocols, layer-based task division, symbol collision resolution |
| ISA Compliance | Thumb-2 IT block migration, immediate encoding constraints, GPIO mask optimization |

END OF TECHNICAL REPORT

ARM Cortex-M4 (nRF52833)  ·  Bare-Metal Assembly  ·  Triple Segment Tree  ·  POV Multiplexer